ELECENG 2EI4

DP #2: Voltage Controlled Switch

Instructor: Dr Haddara

Jasmine Dosanjh – dosanj5 – 400531879

March 2, 2025

Table of Contents

Table of Figures	2

Summary	2

Test Plan	3

Non-ideality 1: When switch is ON it has an internal resistance RON	3

Non-ideality 2: When switch is OFF it has a small leakage current IOFF	3

Non-ideality 3: Vmin<V1 ,  V2<Vmax	4

Non-ideality 4: When switch is ON, RON remains the same with input V1 or V2 (bidirectionality).	4

Switch Type 1	5

i.	Circuit Schematic	5

ii.	Measurements Performed (Test Plan)	5

iii.	Theoretical Explanation	6

iv.	Design Tradeoffs	6

Switch Type 2	7

i.	Circuit Schematic	7

ii.	Measurements Performed (Test Plan)	8

iii.	Theoretical Explanation	8

iv.	Design Tradeoffs	8

References	10

Table of Figures

Figure 1. Switch 1 Schematic	5

Figure 2. Switch 2 Schematic	6

# Summary

# Test Plan

Listed below are experiments to be conducted for each non-ideality. This will identify the performance of the design.

## Non-ideality 1: When switch is ON it has an internal resistance RON

Set Values:

- Vcontrol=0 (to keep the switch closed)

- V1 = 3V

- RL=4400kΩ

Measure:

- V2 (voltage drop across RL)

Calculate:

- RON (voltage drop across closed switch) by voltage division:

V2=V1RLRON+RL

V2=3V1kΩRON+1kΩ

RON=3V(1kΩ)V2-1kΩ

## Non-ideality 2: When switch is OFF it has a small leakage current IOFF

Set Values:

- Vcontrol=5V (to keep the switch open)

- V1 = 3V

- RL=2.2MΩ

Measure:

- V2 (voltage drop across RL)

- IOFF (current through open switch)

  - Use math channel: I=V2-V1RL  <-- OR TRY BOTH  I=V2RL

## Non-ideality 3: Vmin<V1 ,  V2<Vmax

Set Values:

- Vcontrol=5V (to keep the switch open)

- Vmax=5V, Vmin=0V

- V1=3V

- RL=1kΩ

Measure:

- V1 (input voltage)

- V2 (output voltage)

## Non-ideality 4: When switch is ON, RON remains the same with input V1 or V2 (bidirectionality).

Set Values:

- Vcontrol=0V (to keep the switch closed)

- V2=3V

- RL=1kΩ

Measure:

- V2 (voltage drop across RL)

Calculate:

- RON in Direction 2 (when polarity is reversed) by voltage division:

V1=V2RLRON+RL

V1=3V1kΩRON+1kΩ

RON=3V(1kΩ)V1-1kΩ

# Switch Type 1

## Circuit Schematic

The circuit for switch 1 is a transmission MOSFET, with an input voltage V1 an output voltage V2. A load is attached to the output for testing purposes. Refer to Figure 1 for the schematic.

For the pMOS, the control voltage is applied to the gate, and the bulk is attached to the highest voltage Vsupply=+5V. Whereas, for the nMOS, the supply voltage is applied to the gate, and the bulk is attached to the lowest voltage (ground).

Figure 1. Switch 1 Schematic

## Measurements Performed (Test Plan) -- Simulation

Non-ideality 1: When switch is ON it has an internal resistance RON

Set Values:

- Vcontrol=0 (to keep the switch closed)

- V1 = 3V

- RL=4400kΩ

Measure:

- V2 (voltage drop across RL)

Calculate:

- RON (voltage drop across closed switch) by voltage division:

RON=3V(1kΩ)V2-1kΩ

RON=3V(1kΩ)2.999V-1kΩ

RON=0.333

Non-ideality 2: When switch is OFF it has a small leakage current IOFF

Non-ideality 3: Vmin<V1 ,  V2<Vmax

Non-ideality 4: When switch is ON, RON remains the same with input V1 or V2 (bidirectionality).

## Theoretical Explanation

## Design Tradeoffs

# Switch Type 2

## Circuit Schematic

The circuit for switch 2, which is shown in.

The circuit for switch 2 is an extension of switch 1. It is a double transmission MOSFET, with the same input voltage V1 but a separate output voltage VA and VB. It also has a CMOS inverter. Once again, a load is attached to the output for testing purposes. Refer to Figure 2 for the schematic.

For the pMOS, the control voltage is applied to the gate, and the bulk is attached to the highest voltage Vsupply=+5V. Whereas, for the nMOS, the supply voltage is applied to the gate, and the bulk is attached to the lowest voltage (ground).

CMOS Inverter:-

- Need Vcontrol and Vcontrol (inverted).

- Vcontrol is the output of a CMOS inverter.

Top switch:-

- This is switch 1.

- Vcontrol goes to pMOS gate.

- Vcontrol goes to NMOS gate.

Bottom switch:-

- This is a duplicate of switch 1.

- The bulk is attached to the lowest voltage (ground).

- Vcontrol  goes to nMOS gate.

- Vcontrol goes to pMOS gate.

Bottom switch

Top switch

CMOS Inverter

Figure 2. Switch 2 Schematic

## Measurements Performed (Test Plan)

Non-ideality 1: When switch is ON it has an internal resistance RON

Non-ideality 2: When switch is OFF it has a small leakage current IOFF

Non-ideality 3: Vmin<V1 ,  V2<Vmax

Non-ideality 4: When switch is ON, RON remains the same with input V1 or V2 (bidirectionality).

## Theoretical Explanation

## Design Tradeoffs

# References