ELECENG 2EI4

DP #2 Research: Voltage Controlled Switch

Instructor: Dr Haddara

Jasmine Dosanjh – dosanj5 – 400531879

February 16, 2025

Table of Contents

Summary	2

Ideal Switches	2

Switch Non-Idealities	3

Test Plan	3

References	6

# Summary

This project involves designing and building a voltage-controlled switch that approximates ideal switch behaviour. Part one focused on researching key characteristics of ideal and non-ideal switches to understand real-world limitations. Using this knowledge, a test plan was developed to quantify switch performance based on voltage limits, resistance, and leakage current. The second phase will involve implementing the design using MOSFETs, diodes, resistors, and capacitors while complying with the given constraints.

# Ideal Switches

An ideal switch has four main properties. First, when the switch is ON, it is closed and behaves like a perfect wire. This means it has zero resistance and, by Ohm’s Law, experiences a voltage drop of zero [1], as seen in (1). This results in the voltage at one end of the switch being equal to the voltage at the other end (V1=V2), with respect to the ground.

| V=RI where R=0 ∴V=0I=0 | (1) |
| --- | --- |

Second, when a switch is OFF, it behaves like an open circuit, preventing any current flowing through it. Therefore, by Ohm’s Law, its resistance is infinite [1] as seen in (2).

| V=RI where I=0 ∴R=V0 =∞ , where V is any finite value | (2) |
| --- | --- |

Third, because an open switch has infinite resistance, there is no limit on the voltage across it [1]. As seen in (2), the voltage can take any finite value. Finally, an ideal switch is bidirectional, meaning that when it is closed, it behaves like a perfect wire, allowing current to flow in either direction [2].

# Switch Non-Idealities

Real switches exhibit four main non-idealities that affect their performance. When closed, real switches have internal non-zero resistance, which leads to a voltage drop across the switch, so that V1≠V2 [3]. This results in power loss and reduced circuit efficiency. The resistance typically ranges from 50mΩ to 100mΩ, depending on factors such as usage, environmental conditions and material properties [4].

Secondly, when open, real switches have a small leakage current ranging from 0.5mA and 1mA [5]. This indicates that the resistance across the switch is very high but finite, allowing for a small voltage drop. Real switches also have voltage limits when open [3], ranging from 125 V to 250 V AC/DC [6] for small switches like the one in this project. Exceeding these limits can damage both the switch and connected components.

Finally, when a switch is on, the resistance across it remains the same regardless of the direction of current flow [7]. This can be seen by Ohm’s law, as it states that the current flowing through a conductor is directly proportional to the voltage drop, and thus resistance of a conductor is constant.

# References

- M. Alshemmary, “Power Electronics,” General Switching Characteristics. [Online]. Available: https://www.uobabylon.edu.iq/eprints/publication_7_25213_6046.pdf [Accessed: Feb 15, 2025].

- “Ideal Switch,” MathWorks. [Online]. Available: https://www.mathworks.com/help/sps/powersys/ref/idealswitch.html [Accessed: Feb 15, 2025].

- M. Robbins, “Switches,” Ultimate Electronics. [Online]. Available: https://ultimateelectronicsbook.com/switches/ [Accessed: Feb 15, 2025].

- “Interfacing Switches and Relays to the Real World in Real Time,” Analog Devices. [Online]. Available: https://www.analog.com/en/resources/technical-articles/interfacing-switches-and-relays-to-the-real-world-in-real-time.html. [Accessed: Feb 15, 2025].

- “Leakage Current in Opened Switch,” VFD Manufacturers. [Online]. Available: http://www.vfds.org/leakage-current-in-opened-switch-699821.html. [Accessed: Feb 15, 2025].

- “North American Voltage Ranges,” Quick 220° Systems. [Online]. Available: https://quick220.com/pages/north-american-voltage-ranges [Accessed: Feb 15, 2025].

- “Do Resistors Have Polarity: Mini Blog Post,” AX Control Inc. [Online]. Available: https://www.axcontrol.com/blog/2021/do-resistors-have-polarity-mini-blog-post/13/10/ [Accessed: Feb 15, 2025].